Social-tie-based Anonymous Routing Protocol Design

In this section, we describe our proposed social-tie-based anonymous routing protocol. The Protocol consists of two phases: reliable neighbor discovery and anonymous data transmission. During reliable neighbor discovery phase, every node records encounters with its neighbors and finds out most reliable neighbors to send the message.

Notation

Ni, node with global id i

Ns, source node

Si(j), Reliance score node i hold for node j

tijk, kth global encounter time when node i received message from node j

F, Reliance function

MSGm,mth message encrypted with Km

Km, key used to encrypted message MSGm,

Mtype, message type = HELLO, DATA or KEY

Tr, threshold for recognizing reliable neighbors from all neighbors

Anonymous Routing

In our routing protocol, we aim to broadcast a message anonymously and to avoid malicious eavesdropping. Anonymity can be achieved by broadcasting of source node. Without concluding the node identifier in the message, neighbor nodes will not recognize the sender unless there is only one node in their transmission range. Malicious eavesdropping can be avoid by our proposed message-key pair sending schema based on reliance table.

The source node message-key pair sending schema is first broadcasting a message with encrypted data to neighbors. Then, after a certain delay, it sends a message with the key coordinated to the encrypted data. In this way, based on the reliance table, only people who trusted by most people can receive both encrypted data and the key to decrypt it.

Reliance Table Creation

In order to avoid malicious node, every node stores the messages and forwards them to its most reliable neighbors once it receives a message. To find those reliable neighbors, each node maintains a Encounter History Table and Reliance Table. Discovery of the most reliable neighbor includes three steps:

- Advertise Hello Message

Each node periodically advertises Hello messages to discover neighbors in its transmission range and to compute their reliance score.

<sender-id, Mtype = Hello>

- Record Encounter Event

When receiving a hello message, the node records the sender-id in header of the message and time it arrived in the Encounter List. In order to achieve this without acknowledge the neighbours who we are. We use global ID to store neighbours’ social score in Social-tie table.

<sender-id, time>

- Compute Reliable Relationship

Encounter History Table: holds every encounter events with each neighbor node.

Reliance Table: maintains every neighbor node with its reliance score connected to current node.

According to social relationship[14], in order to send message to a certain receiver in human networks, people intend to find others that are more popular than themselves to forward the message. Similarly, when people want to send out information, they always share it with ones they trust most. First, I consider a person reliable if he has connection with me. Such as, we worked together for a class project or we were in the same department. Then, frequency and freshness are taking into consideration. If two people meet frequently, they are more likely to have a strong relationship. Moreover, if a person recently got in touch with me, I can be more familiar with him than others. Thus, reliable relationship is derived from encounter history of a node with its neighbors.

<id, time> is used to illustrate encounter event. R is a score used to evaluate reliable relationship. Assume the global encounter history starting with global time 0, the reliance score of node i relationship with node j at time tcurr denoted by Rij is defined as

Ri(j) = Σk=1,KjF(tjk),

where F(x)is a weighing function to calculate freshness score. As what we illustrated before, F(x)should be monotone decreasing function. Kj is the total number of encounter events with node j. tjk is the global time the kth encounter event between node i with node j.

We take F(x) = (1/2)λx, where λ is a control parameter and 0<=λ<=1 as the weighing function.

Adversary issue:

There can be malicious cheating in this phase. If malicious nodes send hello messages more frequently than ordinary nodes, they are more likely to gain trust from their neighbors due to our reliance computation function. In consideration of this case. we require source nodes to only record one encounter for each neighbor node after they send neighbor request message, Hello message, in step 2.

Message Delivery

When a source node Ns schedule to broadcast information to the whole network anonymously and confidentially, it requires both broadcasting and encryption mechanism due to our analysis.

- Encrypted Message Creation

<Dest,Mtype, Encrypted data>

- Key Message Creation

SHA-2 encryption mechanism is utilized in our design to avoid key confliction.

<Dest,Mtype, key>

- Message Broadcasting

To decrease the possibility that a malicious receive both key and encrypted message, when sending Key Message, source node Ns requires moving around to meet a group of neighbors that is different from the one that it broadcasted Encrypted Message to. That is because when there are same nodes in both group, it is more likely that those two group have same reliable nodes which eliminates our assumption and thus makes our encryption useless. In consider of this movement requirement, a delay tolerance should be introduced into our network. Sending the key and message separately guarantees that no malicious node will get both the key and the encrypted message to decode the plain text during the broadcasting step and the more popular guys among popular guys can get both the key and encrypted message and thus the plain message. Thus, each node Ni has to maintain two more tables:

Encrypted Message Table

Key Message Table

Upon receiving a message, node Ni proceeds with following actions:

- Ni analyzes the message, get message type, key hash id and data

- If message type is Hello message, Ni inserts this encounter event into Encounter History Table and renews Reliance Table.

- If message type is Encrypted message or Key message, Ni searches its key hash id in its existing Message-key Matching Table.

- If key hash id does not exist in the table, which reflects that Ni has never received this packet or its corresponding key/message before, Ni first stores the id and corresponding packet in the table. Then, it finds most reliable neighbors whose reliance score is above a certain threshold in Reliance Table and sends message to those nodes. To implement unicast-like message forwarding, Ni replaces reliableID of the message with its reliable neighbors’ IDs and then broadcast.

- If key hash id exists in the table: if the already stored message is the same as current receiving message,  Ni simply drops the packet. This means that Ni receives the same packet as before. If they are different, Ni uses data in key message to decrypt encrypted message and thus can get the plain data. In both cases, Ni doesn’t broadcast the message it receives to anyone else.

Adversary Issue:

Malicious node can ignore the protocol and broadcast every message to its neighbors in order to let more malicious nodes receive the message and thus increase possibility to get both key and corresponding encrypted message.

A

B

C

D

E

F

A’

B’

message

key

The bad case:

A

B

B has only one link which is A. When B receive an encrypted message, it will definitely know the source is A because only if A is less popular than B will A send the message to B, so A has at most one connection in its range which is B. However, even though B receives this message, he cannot know the context of the message (Is this anonymous or not?).

Considering this case: we don’t allow the source to send message when there is only one node in its neighbour.

A:a

B:b

C:c

D:d

Things happen similar with multiple nodes. upper case alphabet indicates the node and the lower ones are their social score. We assume d > c > b > a. Then when B receives the message, it will definitely know that it is A sending to him.

Thus it seems more popular the source node, more anonymous the network will be.